Presentation Title

Development and Filed Evaluation of a Texture Measurement System Based on Continuous Profiles from a 3D Scanning System

Yaxiong (Robin) Huang1, Todd Copenhaver2, Phillip Hempel3,  Magdy Mikhail4,

Pavement Preservation Section, Maintenance Division, Texas Department of Transportation

ABSTRACT

Pavement texture is an important indicator of road serviceability. Pavement texture is directly linked to the pavement and tire skid resistance and traffic safety to the public. Therefore, most transportation agencies across the nation require texture data for their Pavement Management Information System.  Current pavement texture measurement methods of using high speed distance sensor or 3D line scan devices experienced limitations on data stability. Problems caused by vehicle vibration, bad data from optical sensors and most importantly, speed dependency related to the sensor sampling time or exposure time make texture result either unreliable or suffering noticeable errors. This paper reports a study to the speed influence on network level texture data collection, and the development of a high speed and high precise 3D texture system. Experiment confirmed that improper selection of a sensor with large exposure time will result in speed dependency on measured texture data. As the system developed in this study adopted a very small exposure time for each sampling, it can produce accurate texture data largely independent from vehicle speed.

Keywords

Pavement texture, 3D texture, speed dependency

.