Validation of Automated Pavement Distress Detection, Evaluation and Reporting Processes – Canadian Experience

Li Ningyuan1 and Stephen Lee2

（1Senior Pavement Management Engineer

Ministry of Transportation of Ontario

1201 Wilson Avenue, Downsview

Ontario, Canada

E-mail: Li.Ningyuan@mto.gov.on.ca）

（2Head of Pavements and Foundations Section

Ministry of Transportation of Ontario

1201 Wilson Avenue, Downsview

Ontario, Canada)

ABSTRACT

Ontario Ministry of Transportation has started to plan for implementing a full scale of automatic high-speed pavement condition data collection, evaluation and reporting processes that support operational management of the provincial pavement network. The data collection system is equipped with Pavemetrics’ Laser Crack Measurement System (LCMS) in addition to longitudinal and transverse profilers that measure pavement roughness and wheel path rutting on pre-defined road sections. The data collected in the field can be processed through Fugro Roadware’s Vision software that can be customized to meet the Ontario provincial pavement performance evaluation and rating requirements, including International Roughness Index (IRI), Surface Distress Index (SDI) and overall Pavement Condition Index (PCI). To ensure quality of the automated pavement condition assessment, a series of comparative measurements in the field were conducted to verify whether each of the stages has been accurately processed, such as pavement distress detection, classification, severity rating and density calculating, condition evaluation and reporting.

The core objective of this study is twofold: 1) to examine the precision and accuracy of the automated process of pavement condition evaluation results in comparison with the ground truth data conducted manually in the field, 2) to  perform the impact analysis of the fully automated process of pavement condition data collection and evaluation results on the current pavement management operation systems in terms of performance evaluation, pavement maintenance and rehabilitation programming and investment planning.

Some of the significant results and conclusions resulting from the study include: 1) comparability analysis summary of the automated pavement surface distress evaluation versus the traditional manual assessment, 2) extended applications of the automated pavement distress detection, rating and evaluation to performance specifications for pavement rehabilitation and maintenance contracts, 3) validation and calibration of pavement performance indices calculated from the automated data collection system to the ground truth measurements, 4) development of the quality assurance of data collection program and procedure applied to Ontario pavement management system at network level.

Keywords: Automated Pavement Condition Surveys, Data Collection, Distress Evaluation, Verification Process, Quality Assurance